C.R. Laurence Co., Inc.		Distributed by ZeroDocs.com

This section includes editing notes to assist the user in editing the section to suit project requirements. These notes are included as hidden text, and can be revealed or hidden by the following method in Microsoft Word:

Display the FILE tab on the ribbon, click OPTIONS, then DISPLAY. Select of deselect HIDDEN TEXT.

This guide specification section has been prepared by C.R. Laurence Co., Inc. for use in the preparation of a project specification covering the Cielo Post Windscreen System windscreen system.

The following should be noted in using this specification:

Hypertext links to manufacturer websites are included after manufacturer names to assist in product selection and further research. Hypertext links are contained in blue, e.g.:

www.crlaurence.com

Optional text requiring a selection by the user is enclosed within brackets and as red text, e.g.: AColor: [Red.] [Black.]"

Items requiring user input are enclosed within brackets and as red text, e.g.: "Section [__ __ __ - ________]."

Optional paragraphs are separated by an "OR" statement included as red text, e.g.:

**** OR ****

For assistance in the use of products in this section, contact C.R. Laurence Co., Inc. by calling 800-421-6144 or visit their website at www.crlaurence.com.

This specification has been prepared based on SimpleSpecs™ specification templates. The SimpleSpecs™ Master Guide Specification system comprises a full architectural master specification that can be used to specify all project requirements. For additional information visit the ZeroDocs.com

SECTION 10 71 23 - WINDSCREENS

- GENERAL

- SUBMITTALS

- Action Submittals:

- Shop Drawings: Illustrate products, installation, and relationship to adjacent construction.

- Product Data: Manufacturer’s descriptive data and product attributes.

- Samples: [Selection samples.] [Verification samples.]

- Informational Submittals:

- Certificate of Compliance: Certification that installed products meet specified design and performance requirements.

- SYSTEM DESCRIPTION

- Design Requirements:

- Cielo can withstand wind loads from 15 to 80 PSF over windscreen dimensions from 2’-6” to 5’-0” inches wide and 6’-0” to 8’-0” high. Glass must be 9/16 inch thick fully tempered and laminated.

- Design wind pressure: In accordance with [ASCE 7.] [Building Code.] [____.]

- Movement: Ambient temperature range of [120] [__] degrees F and a surface temperature range of [160] [__] degrees F.

- System design to be performed by qualified professional engineer licensed in State of [____].

- QUALITY ASSURANCE

Retain the following to specify minimum installer qualifications.

- Installer Qualifications: Firm specializing in work of this Section with minimum [2] [__] years’ experience.

Retain the following for a full-size mockup.

- Mockup: [Minimum [4] [__] feet long x full height.] [____.] Approved mockup [may] [may not] remain as part of the Work.

- PRODUCTS

- MANUFACTURERS

- Contract Documents are based on Cielo Post Windscreen System by C.R. Laurence Co., Inc. www.crlaurence.com

- Edit the following to indicate whether substitutions will be allowed for the products in this section.

- Substitutions: [Refer to Division 01.] [Not permitted.]

- MATERIALS

- Aluminum Extrusions: ASTM B221, 6005-T6 alloy and temper.

In accordance with the IBC, glass has been deigned using a safety factor of 4 in determining allowable flexural stress (6000 PSI).

- Glass: ASTM C1172 and ANSI Z97.1, nominally 9/16 inch thick [clear] [[____] tinted] laminated with exposed edges polished, consisting of two layers of ¼ inch thick fully tempered glass with 0.060 inch interlayer.

- FABRICATION

- Posts are available up to 8 feet in height with a maximum spacing of 5 feet on center.

    - Windscreen Height: [[6] [7] [8] [__] feet.] [As indicated.]

    - Posts:

      - [End] [and] [center] posts: 3-1/4 inch front trim x 4-1/2 inch depth.

      - Corner posts: 4-1/2 x 4-1/2 inches, 90 degree.

    - Horizontal Extrusions: Not required.

    - Mounting: [Surface.] [Core.]

- FINISHES

- Aluminum: Powder coating, AAMA 2604, [Matte Black] [Silver Metallic] [Sky White] [custom] color [to be selected from manufacturer's full color range].

- EXECUTION

Anchoring schedules and recommendation shall be made by a license structural engineer.

Concrete slab shall be 4000 PSI normal weight, 6 inch minimum thickness, for surface mount stanchion and 8 inch minimum thickness for embedded stanchions. Cracked concrete. lightweight concrete slab,  and corrugated metal deck slabs are not acceptable.

- INSTALLATION

- Install in accordance with manufacturer's instructions and approved Shop Drawings.

- Installation Tolerances:

- Maximum variation from plumb or level: [1/16] [__] inch in 3 feet or [1/8] [__] inch in any 10 feet, whichever is less.

END OF SECTION