ALUMINUM TICKET WINDOW

SECTION 08580

# PART 1 – GENERAL

1.01 SUMMARY

- This section includes:

- Aluminum ticket windows as indicated in drawings and in sections.

1.02 SUBMITTALS

- Product Data: Submit Manufacturer’s technical product data substantiating that products comply.

- Shop drawings: Submit for fabrication and installation of windows. Include details, elevations and installation requirement of finish hardware and cleaning.

C.	Certification: Provide printed data in sufficient detail to indicate compliance with the contract documents.

1.03 DELIVERY, STORAGE, AND HANDLING

- Deliver windows crated to provide protection during transit and job storage

- Inspect windows upon delivery for damage. Unless minor defects can be made to meet the Architect’s specifications and satisfaction, damaged parts should be removed and replaced.

- Store windows at building site under cover in dry location.

1.04 PROJECT CONDITIONS

- Field measurements: Check opening by accurate field measurement before fabrication. Show recorded measurements on shop drawings. Coordinate fabrication schedule with construction progress to avoid delay of work.

1.05 WARRANTY

All material and workmanship shall be warranted against defects for a period of one (1)

year from the original date of purchase.

ALUMINUM TICKET WINDOW

# SECTION 08580 -2

# PART 2 - PRODUCTS

2.01 ACCEPTABLE MANUFACTURER’S

- Basis of design: Design is based on Aluminum Ticket Window, catalog number SCW102N,  manufactured by C.R. Laurence Co., Inc. (800) 421-6144

2.02 MATERIALS

- Frames: Aluminum ticket window frame to be 1.390” x .625” extruded aluminum. Overall frame size to be 24” W x 38”H (Includes 2” H stainless steel shelf.)

- Finish: All aluminum to be clear anodized.

Glazing: ¼” Clear tempered Glass.

- Shelf: Provide a shelf not less than 2” thick. The shelf is to be the full width of the window and 18” deep centered under the glazing.

- Voice Transmission: Communication permitted by 834A no draft speak-thru centered in glazing.

- Ticket Window: Half-round ticket window model 720A to be centered in glazing, and sits atop stainless steel shelf.

PART 3 – EXECUTION

3.01 INSTALLATION

- Install frames and glazing in accordance with manufacturer’s printed instructions and recommendations. Repair damaged units as directed (if approved by the manufacturer and the architect) or replace with new units.

3.02 CLEANING

A.	Clean frame and glazing surfaces after installation, complying with requirements contained in the manufacturer’s instructions. Remove excess glazing sealant compounds, dirt or other substances.

3.03 PROTECTION

A.	Institute protective measures required throughout the remainder of the construction period to ensure that all the windows do not incur any damage or deterioration, other than normal weathering, at the time of acceptance.

## END OF SECTION