“SHARYN”  INTERIOR SLIDING PASS WINDOWS

SECTION 08 56 19

# PART 1 – GENERAL

1.1 SUMMARY

- This section includes:

- Frameless interior pass-thru sliding service windows as indicated in drawings and in sections.

1.2 RELATED REQUIREMENTS

A.	Window Finish Color: Section 09 06 00, SCHEDULE FOR FINISHES.

B.	Countertop Construction: Section 09 20 00, FINISH CARPENTRY.

C.	Glass and Glazing: Section 08 80 00, GLAZING.

1.3 SUBMITTALS

- Submittal Procedures: Section 01 33 23, SHOP DRAWINGS, PRODUCT DATA, AND SAMPLES.

- Shop Drawings: Submit for fabrication and installation of windows. Include details, elevations and installation requirement of finish hardware and cleaning.

- Product Data: Submit Manufacturer’s technical product data substantiating that products comply, and installation instructions.

- Samples: Exposed finishes. 4” (102 mm) piece of each color.

- Sustainable Construction Submittals: Recycled content. Identify post-consumer and pre-consumer recycled content percentage by weight.

1.4 DELIVERY, STORAGE, AND HANDLING

- Deliver windows crated to provide protection during transit and job storage.

- Inspect windows upon delivery for damage. Unless minor defects can be made to meet the Architect’s specifications and satisfaction, damaged parts should be removed and replaced.

- Store windows at building site under cover in dry location.

- Protect products from damage during handling and construction operations.

1.5 WARRANTY

A.	Warranty: All material and workmanship shall be warranted against defects for a period of one (1) year from the original date of purchase.

# PART 2 – PRODUCTS

2.1 MATERIALS

- Header: Shall be constructed of 6063-T5 extruded aluminum.  Window rolls on top-hung ball bearing rollers. Overall size is to be in accordance with the contract drawings.

- Finish: All aluminum to be brite anodized, satin anodized, or bronze anodized (Specify).

- Glazing: The glazing is to be ¼” (6 mm) in thickness.

- Options: Keyed lock, push button lock, fixed sidelite header insert, surface mounted bottom track.

- Models:  Sharyn (OX), (XO), and (XX). X = sliding panel, O = fixed panel, as viewed from clerks side.

2.2 PRODUCTS GENERAL

- Basis of design: Design is based on Sharyn Series Frameless Interior Pass-Thru Window manufactured by:

C.R. Laurence Co., Inc. (800) 421-6144 Ext. 7760

transaction@crlaurence.com

2.3 PASS WINDOWS

        - Pass Window Units: Factory fabricated, glazed unit; horizontal sliding type.

          - Header: Extruded aluminum.

          - Glass: Safety type specified in Section 08 80 00, GLAZING.

          - Hardware: Manufacturer’s standard double track header, rollers, guides, push button lock.

PART 3 – EXECUTION

3.1 PREPARATION

- Examine and verify substrate suitability for product installation.

- Verify rough opening is properly sized and located.

- Protect existing construction and completed work from damage.

- Apply barrier coating to aluminum surfaces in contact with dissimilar metals and cementitious materials to minimum 0.7 mm (30 mils) dry film thickness.

3.2 INSTALLATION

- Install window in accordance with manufacturer’s printed instructions and recommendations. Repair damaged units as directed (if approved by the manufacturer and the architect) or replace with new units.

3.3 CLEANING

A.	Clean frame and glazing surfaces after installation, complying with requirements contained in the manufacturer’s instructions. Remove excess glazing sealant compounds, dirt or other substances.

3.4 PROTECTION

A.	Institute protective measures required throughout the remainder of the construction period to ensure that all the windows do not incur any damage or deterioration, other than normal weathering, at the time of acceptance.

## END OF SECTION