STAINLESS STEEL BULLET RESISTANT PACKAGE RECEIVER

SECTION 13070

# PART 1 – GENERAL

1.01 SUMMARY

- This section includes:

- Stainless Steel bullet resistant package receivers as indicated in drawings and in sections.

- The publication below forms a part of this specification:

1.UNDERWRITERS LABORATORY UL 752 9th Edition

Standard for Bullet Resisting Equipment dated Jan. 27, 1995.

1.02 SUBMITTALS

- Product Data: Submit Manufacturer’s technical product data substantiating that product comply.

- Shop drawings: Submit for fabrication and installation of windows. Include details; elevations and installation requirement of finish hardware and cleaning.

C.	Certification: Provide printed data in sufficient detail to indicate compliance with the contract documents.

1.03 DELIVERY, STORAGE, AND HANDLING

- Deliver package receiver boxed to provide protection during transit and job storage

- Inspect package receiver upon delivery for damage. Unless minor defects can be made to meet the Architect’s specifications and satisfaction, damaged parts should be removed and replaced.

- Store package receiver at building site under cover in dry location.

1.04 PROJECT CONDITIONS

- Field measurements: Check opening by accurate field measurement before fabrication. Show recorded measurements on shop drawings. Coordinate fabrication schedule with construction progress to avoid delay of work.

1.05 WARRANTY

All material and workmanship shall be warranted against defects for a period of one (1)

year from the original date of purchase.

STAINLESS STEEL BULLET RESISTANT PACKAGE RECEIVER

SECTION 13070-2

# PART 2 - PRODUCTS

2.01 ACCEPTABLE MANUFACTURER’S

- Basis of design: Design is based on Stainless Steel Bullet Resistant Package Receiver with Vision Panel Door, # SPRVP1 (level 1) or SPRVP3 (level 3) manufactured by C.R. Laurence Co., Inc. (800) 421-6144 x7760 Technical Sales.

2.02 MATERIALS

- Frames: Stainless Steel bullet resistant frame modules shall be to the standards established by U.L. 752 Protection Level 1 or 3.  Frames are to be constructed of 11 ga. stainless steel.  (7 ga. stainless steel doors on level 3 doors only), # 4 finish.  All joints are mitered and welded.  All exposed welds shall be ground flush and finished smooth. Sizes are to be in accordance with the contract drawings 18-1/2” W x 18-1/2” H x 16” D (Outside Dimensions.) FRAMES MUST UTILIZE TESTING RECOGNIZED UNDER THE STANDARDS ESTABLISHED BY U.L. 752 FOR BULLET RESISTANT COMPONENTS.

- Finish: # 4 stainless steel (On doors only)

- Glazing: The glazing must be in accordance with U.L. 752 testing standards

Level 1 (1-1/4” Acrylic) Level 3, (1-1/4”  Polycarbonate.) (Vision Panel Door.)

D.        Door interlocking mechanism, automatic door closer on threat side, and lockout for the threat side controlled from safe side, installation flanges supplied as standard by manufacturer.

PART 3 – EXECUTION

3.01 INSTALLATION

- Install package receiver in accordance with manufacturer’s printed instructions and recommendations. Repair damaged units as directed (if approved by the manufacturer and the architect) or replace with new units.

STAINLESS STEEL BULLET RESISTANT PACKAGE RECEIVER

SECTION 13070-3

3.02 CLEANING

A.	Clean frame and glazing surfaces after installation, complying with requirements contained in the manufacturer’s instructions. Remove excess glazing sealant compounds, dirt or other substances.

3.03 PROTECTION

A.	Institute protective measures required throughout the remainder of the construction period to ensure that the package receivers do not incur any damage or deterioration, other than normal weathering, at the time of acceptance.

## END OF SECTION