Manufacturer:

C.R. Laurence

2450 E. Vernon Ave.

Los Angeles, California 90058-1802

Toll Free Phone: (800) 421-6144

Toll Free Fax: (866) 262-3299

SECTION 08 11 16

ENTICE CLADDED ALUMINUM FRAMED ALL-GLASS Entrance Doors

PART 1: GENERAL

1.01	SECTION INCLUDES

A.	Aluminum-Framed All Glass Entrance Doors

B.	Infill panels of glass

C.	Aluminum doors and frames

D.	Weatherstripping

E.	Door hardware

F.	Perimeter sealant

1.02	RELATED REQUIREMENTS

A.	Section ______-____________: Preparation of adjacent work to receive work of this section

B.	Section 05 1200 - Structural Steel Framing: Steel attachment members

C.	Section 05 5000 - Metal Fabrications: Steel attachment devices

D.	Section 07 2500 - Weather Barriers: Perimeter air and vapor seal between glazing system and adjacent construction

E.	Section 07 9005 - Joint Sealers: Perimeter sealant and back-up materials

F.	Section 08 7100 - Door Hardware: Hardware items other than specified in this section

G.	Section 08 8000 - Glazing: Glass and glazing accessories

H.	Section 08 4413 - Glazed Aluminum Curtain Walls

1.03	REFERENCE STANDARDS

A.	AAMA CW-10 - Care and Handling of Architectural Aluminum from Shop to Site; American Architectural Manufacturers Association; 2012

B.	AAMA 501.2 - Field Check of Metal Storefronts, Curtain Walls, and Sloped Glazing Systems for Water Leakage; American Architectural Manufacturers Association; 2009 (part of AAMA 501)

C.	AAMA 1503 - Voluntary Test Method for Thermal Transmittance and Condensation Resistance of Windows, Doors and Glazed Wall Sections; American Architectural Manufacturers Association; 2009

D.	AAMA 2605 - Voluntary Specification, Performance Requirements and Test Procedures for Superior Performing Organic Coatings on Aluminum Extrusions and Panels; 2011

E.	ASTM B221 - Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes; 2012

F.	ASTM B221M - Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes [Metric]; 2012

1.04	ADMINISTRATIVE REQUIREMENTS

A.	Coordinate with installation of other components that comprise the exterior enclosure

1.05	SUBMITTALS

A.	See Section 01 3000 - Administrative Requirements, for submittal procedures

B.	Product Data: Provide component dimensions, describe components within assembly, anchorage and fasteners, glass and infill, door hardware, internal drainage details and ________

C.	Shop Drawings: Indicate system dimensions, framed opening requirements and tolerances, affected related Work, expansion and contraction joint location and details, and field welding required

D.	Hardware Schedule: Complete itemization of each item of hardware to be provided for each door, cross-referenced to door identification numbers in Contract Documents

E.	Warranty: Submit manufacturer warranty and ensure forms have been completed in Owner's name and registered with manufacturer

1.06	QUALITY ASSURANCE

A.	Manufacturer and Installer Qualifications: Company specializing in manufacturing aluminum glazing systems with minimum three years of documented experience

1.07	DELIVERY, STORAGE, AND HANDLING

A.	Handle products of this section in accordance with AAMA CW-10

B.	Protect finished aluminum surfaces with wrapping. Do not use adhesive papers or sprayed coatings that bond to aluminum when exposed to sunlight or weather

1.08	FIELD CONDITIONS

A.	Do not install sealants when ambient temperature is less than 40 degrees F (5 degrees C). Maintain this minimum temperature during and 48 hours after installation

1.09	WARRANTY

A.	See Section 01 7800 - Closeout Submittals, for additional warranty requirements

PART 2: PRODUCTS

2.01	BASIS OF DESIGN -- SWINGING DOORS

A.	Ultra Narrow Stile, Fully Assembled with Insulating Glazing, Thermally Broken:

- Basis of Design: C.R. Laurence Co., Inc; Entice Series 104

- Basis of Design: C.R. Laurence Co., Inc, Entice Series 110

Stiles: 1-1/8 inches (28.5 mm) Ultra Narrow Stile Thermal Entrance Door.

Thickness: 2-1/2 inches (57.1 mm).

Top Rail: 4”

Bottom Rail: 4” Series 104

Bottom Rail: 10” Series 110

Intermediate Vertical: 2”

D.	Other Manufacturers: Provide either the product identified as "Basis of Design" or an equivalent product of one of the manufacturers listed below:

1.	C.R. Laurence Co., Inc; www.crlaurence.com

E.	Substitutions: See Section 01 6000 - Product Requirements.

1.	For any product not identified as "Basis of Design", submit information as specified for substitutions.

2.02	MANUFACTURERS

A.	Aluminum-Framed Storefront and Doors:

1.	C.R. Laurence Co., Inc; www.crlaurence.com.

2.	Substitutions: See Section 01 6000 - Product Requirements.

2.03	STOREFRONT

A.	Aluminum-Ultra Framed Storefront: Factory fabricated, factory finished aluminum framing members for insulated glazing, and related flashings, anchorage, and attachment devices.

1.	Glazing Position: Centered (front to back).

2.	Vertical Mullion Dimensions: 1-1/4” inch face dimension by 2-9/16” inch depth (31.7 mm wide by 65 mm).

3.	Finish: Class I color anodized.

a.	Factory finish all surfaces that will be exposed in completed assemblies.

b.	Touch-up surfaces cut during fabrication so that no natural aluminum is visible in completed assemblies, including joint edges.

c.	Coat concealed metal surfaces that will be in contact with cementitious materials or dissimilar metals with bituminous paint.

4.	Fabrication: Joints and corners flush, hairline, and weatherproof, accurately fitted and secured; prepared to receive anchors and hardware; fasteners and attachments concealed from view; reinforced as required for imposed loads.

5.	Construction: Eliminate noises caused by wind and thermal movement, prevent vibration harmonics, and prevent "stack effect" in internal spaces.

6.	System Internal Drainage: Drain to the exterior by means of a weep drainage network any water entering joints, condensation occurring in glazing channel, and migrating moisture occurring within system.

7.	Expansion/Contraction: Provide for expansion and contraction within system components caused by cycling temperature range of 170 degrees F (95 degrees C) over a 12-hour period without causing detrimental effect to system components, anchorages, and other building elements.

8.	Movement: Allow for movement between storefront and adjacent construction, without damage to components or deterioration of seals.

9.	Perimeter Clearance: Minimize space between framing members and adjacent construction while allowing expected movement.

2.04	MATERIALS

A.	Extruded Aluminum: ASTM B221 (ASTM B221M).

B.	Perimeter Sealant: Type _____ specified in Section 07 9005.

C.	Glazing Accessories: As specified in Section 08 8000.

2.05	FINISHES (ARCHITECT TO INCLUDE ONE OPTION FROM BELOW):

A.	Class I Natural Anodized Finish: AAMA 611 AA-M12C22A41 Clear anodic coating or AAMA 612 clear anodic coating with electrolytically deposited organic seal; not less than 0.7 mils (0.018 mm) thick. Stainless Steel and Painted Finishes also Available. [specify]

B.	Superior Performance Organic Coating System: AAMA 2605 multiple coat, thermally cured polyvinylidene fluoride system.

C.	Color: As shown on the drawings.

D.	Color: To be selected by Architect from manufacturer's standard range.

E.	Touch-Up Materials: As recommended by coating manufacturer for field application.

  - HARDWARE

- Locks: Equip exterior doors with manufactures lockset compatible with 2-1/2” thick door

  - Location and function: Provide round throw deadbolt in continues bottom fitting.

  - Lock to be operated by key outside and thumb turn inside with end load capability.

B.    Cylinders or Magnetic Locks: Consult with Factory

C.    Deadbolt Lock Handles: Shall be C.R. Laurence’s DB100, DB110, DB130, DB140, DB150, DB160, DB170

D.    Concealed Vertical Rod Panics: Shall be 1-1/4” diameter C.R. Laurence’s PA100 or PA110

- Push/Pull Hardware: Shall be provided by C.R. Laurence and selected by architect.

- Medium and Heavy-Duty Floor Closers:  CRL/J990, Rixon 27 or 28, Dorma BTS80, … with Top Pivot

- Heavy Duty OHCC: Shall CRL 9200 Series.

Arms can be secured to top or bottom rails; all Non-Hold Open Closers must comply with A.D.A. requirements.

- Electronic Egress Control Handles: Shall be C.R. Laurence’s EG100 or EG110

- Electric Strike: Shall be Folger Adams 310-1

J.	Weatherstripping: CRL Fin, continuous and replaceable; provided on all doors.

K.	Threshold: Extruded aluminum, one piece per door opening, ribbed surface; provide on All Doors.

L.    Adjusting Mechanism: Shall be CRL Kwick-Adjust Mechanism

M.     All Hardware must comply with Manufactures specifications items include closer size, exposure to weather, and traffic conditions [light to moderate] [moderate to heavy] [heavy] select one.

PART 3: EXECUTION

3.01	EXAMINATION

A.	Verify dimensions, tolerances, and method of attachment with other work.

B.	Verify that wall openings and adjoining air and vapor seal materials are ready to receive work of this section.

3.02	INSTALLATION

A.	Install wall system in accordance with manufacturer's instructions.

B.	Attach to structure to permit sufficient adjustment to accommodate construction tolerances and other irregularities.

C.	Provide alignment attachments and shims to permanently fasten system to building structure.

D.	Align assembly plumb and level, free of warp or twist. Maintain assembly dimensional tolerances, aligning with adjacent work.

E.	Provide thermal isolation where components penetrate or disrupt building insulation.

F.	Install sill flashings. Turn up ends and edges; seal to adjacent work to form watertight dam.

G.	Where fasteners penetrate sill flashings, make watertight by seating and sealing fastener heads to sill flashing.

H.	Coordinate attachment and seal of perimeter air and vapor barrier materials.

I.	Pack fibrous insulation in shim spaces at perimeter of assembly to maintain continuity of thermal barrier.

J.	Set thresholds in bed of mastic and secure.

K.	Install hardware using templates provided.

1.	See Section 08 7100 for hardware installation requirements.

L.	Install glass and infill panels in accordance with Section 08 8000, using glazing method required to achieve performance criteria.

M.	Install perimeter sealant in accordance with Section 07 9005.

N.	Touch-up minor damage to factory applied finish; replace components that cannot be satisfactorily repaired.

3.03	TOLERANCES

A.	Maximum Variation from Plumb: 0.06 inches every 3 ft (1.5 mm/m) non-cumulative or 1/16 inches per 10 ft (1.5 mm/3 m), whichever is less.

B.	Maximum Misalignment of Two Adjoining Members Abutting in Plane: 1/32 inch (0.8 mm).

3.04	ADJUSTING

A.	Adjust operating hardware and sash for smooth operation.

3.05	CLEANING

A.	Remove protective material from pre-finished aluminum surfaces.

B.	Wash down surfaces with a solution of mild detergent in warm water, applied with soft, clean wiping cloths. Take care to remove dirt from corners. Wipe surfaces clean.

C.	Remove excess sealant by method acceptable to sealant manufacturer.

3.06	PROTECTION

A.	Protect installed products from damage during subsequent construction.

END OF SECTION