TRANSPARENT BULLET RESISTANT BARRIER SYSTEM

PLASTIC GLAZING

SECTION 08840

# PART 1 – GENERAL

1.01 SUMMARY

- This section includes:

- Transparent bullet resistant barrier system as indicated in drawings and in sections.

- The publication below forms a part of this specification:

1.UNDERWRITERS LABORATORY UL 752 9th Edition

Standard for Bullet Resisting Equipment dated Jan. 27, 1995.

1.02 SUBMITTALS

- Product Data: Submit Manufacturer’s technical product data substantiating that products comply.

- Shop drawings: Submit for fabrication and installation of barrier system. Include details, elevations and installation requirement of finish hardware and cleaning.

C.	Certification: Provide printed data in sufficient detail to indicate compliance with the contract documents.

1.03 DELIVERY, STORAGE, AND HANDLING

- Deliver materials crated to provide protection during transit and job storage

- Inspect materials upon delivery for damage. Unless minor defects can be made to meet the Architect’s specifications and satisfaction, damaged parts should be removed and replaced.

- Store materials at building site under cover in dry location.

1.04 PROJECT CONDITIONS

- Field measurements: Check opening by accurate field measurement before fabrication. Show recorded measurements on shop drawings. Coordinate fabrication schedule with construction progress to avoid delay of work.

1.05 WARRANTY

All material and workmanship shall be warranted against defects for a period of one (1)

year from the original date of purchase.

TRANSPARENT BULLET RESISTANT BARRIER SYSTEM

PLASTIC GLAZING

# SECTION 08840- 2

# PART 2 - PRODUCTS

2.01 ACCEPTABLE MANUFACTURERS

- Basis of design: Design is based on Transparent Bullet Resistant Barrier System, manufactured by C.R. Laurence Co., Inc. (800) 421-6144

2.02 MATERIALS

- Clamps: #304, 11 gauge Stainless Steel. Finishes available, Brushed Stainless, Polished Stainless, and Satin Chrome. (Specify finish).   Replacement of glazing shall be from the secure side of the window or wall unit. Shapes and sizes are to be in accordance with the contract drawings. Stainless Steel U-Channel 11 gauge over each deal tray and base per basis of design.

- Snap Sash: Constructed of 6063-T5 extruded aluminum alloy with matching end caps. Two-piece snap sash for 1-1/4” glazing.

- Finish: All aluminum to be satin anodized, or duranodic bronze (Specify color).

- Glazing: The glazing must be in accordance with U.L. 752 testing standards

Level ____ (specify level 1 or 3). 1-1/4” thick acrylic level 1 or 1-1/4” thick polycarbonate level 3 (Lexgard.) All exposed edges to be polished.

- Voice Transmission: Communication permitted by one of the following as selected by architect. (Options include: #N666 round 6”stainless steel speak thru, #CAD101 round 10” clear acrylic, #CLD103 round 10” clear Lexgard, “Surround Sound” which provides for natural voice communication permitted by the design and use of surround sound spacers with side buttresses , or electronic thru glass communicator # TTU1, with optional headset.)

PART 3 – EXECUTION

3.01 INSTALLATION

- Install clamps, snap sash, plastic glazing, and any other materials in accordance with manufacturer’s printed instructions and recommendations. Repair damaged units as directed (if approved by the manufacturer and the architect) or replace with new units.

TRANSPARENT BULLET RESISTANT BARRIER SYSTEM

PLASTIC GLAZING

SECTION 08840 -3

3.02 CLEANING

A.	Clean plastic glazing surfaces after installation, complying with requirements contained in the manufacturer’s instructions. Remove excess glazing sealant compounds, dirt or other substances.

3.03 PROTECTION

A.	Institute protective measures required throughout the remainder of the construction period to ensure that all the plastic glazing panels do not incur any damage or deterioration, other than normal weathering, at the time of acceptance.

## END OF SECTION