SECTION 10 71 13

EXTERIOR SUN SHADES

USE THIS SECTION WHEN SPECIFYING C.R. LAURENCE 7700 SERIES ALUMINUM SUN SHADE SYSTEMS.      THE CRL 7700 SERIES SUN SHADE SYSTEM IS A FIXED BAR GRILLE STYLE WITH A TAPERED BULL NOSE EDGE, FABRICATED IN THREE (3) STANDARD LENGTHS TO MINIMIZE FIELD LABOR AND CUT DOWN ON COST.    UNITS ARE SHIPPED ASSEMBLED FOR EASY INSTALLATION.    CUSTOM LENGTHS ARE AVAILABLE UPON REQUEST.    ALL STANDARD SIZE UNITS COME PRE-FINISHED IN A METALLIC SILVER COLOR POWDER COAT FINISH.

THIS SPECIFICATION SECTION IS A MANUFACTURER SPECIFIC PRODUCT SPECIFICATION USING THE PROPRIETARY METHOD OF SPECIFYING APPLICABLE TO PROJECT SPECIFICATIONS AND MASTER GUIDE SPECIFICATIONS.    THIS SPECIFICATION SECTION SHOULD BE EDITED TO MEET SPECIFIC PROJECT DESIGN CRITERIA BY A KNOWLEDGEABLE CONSTRUCTION SPECIFIER.    OPTIONS ARE SHOWN IN BRACKETS [ ]. CHOOSE OPTIONS THAT MEET DESIGN CRITERIA, AND REMOVE BRACKETS AND UNUSED OPTIONS BEFORE PRINTING.

PART 1    GENERAL

1.01	SECTION INCLUDES

A.	Manufactured architectural aluminum exterior sun shades; fixed bar grille style with tapered bull nose edge, mechanically attached to exterior wall systems listed below:

1.	Aluminum curtain wall.

2.	Pre-cast concrete.

3.	Metal panel.

4.	Aluminum Storefront.

5.	Concrete tilt-up.

1.02	RELATED REQUIREMENTS

A.	Section 05 40 00 - Cold-Formed Metal Framing:    Backing Supports, attachment, clips, and blocking to receive sunshades.

B.	Section 08 44 00 Curtain Wall -    exterior mounting surface to receive sun shades

C.	Section 07 42 13    Metal Wall Panels - exterior mounting surface to receive sun shades.

1.03	REFERENCE STANDARDS

A.	AAMA 2605 - High Performance Organic Coatings on Architectural Extrusions and Panels.ASTM A 276 - Standard Specification for Stainless Steel Bars and Shapes.

B.	ASTM A 240 - Stainless Steel Sheet and Plate.

C.	ASTM A 276 - Stainless Steel Bars and Shapes.

D.	ASTM A 480 - General Requirements for Flat-Rolled Stainless and Heat-Resisting Steel Plate, Sheet, and Strip.

E.	ASTM A 666 - Annealed or Cold-Worked Austenitic Stainless Steel Sheet, Strip, Plate, and Flat Bar.

F.	 ASTM B117 -    Standard Practice for Operating Salt Spray (Fog) Apparatus.

G.	ASTM B 209 - Aluminum and Aluminum-Alloy Sheet and Plate.

H.	ASTM B 221 - Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

I.	ASTM D822 - Standard Practice for Filtered Open-Flame Carbon-Arc Exposures of Paint and Related Coatings.

J.	ASTM D2244 - Standard Test Method for Calculation of Color Differences From Instrumentally Measured Color Coordinates.

K.	ASTM D2794 - Standard Test Method for Resistance of Organic Coatings to the Effects of Rapid Deformation (Impact).

L.	ASTM D3363 - Standard Test Method for Film Hardness by Pencil Test.

M.	ASTM D4214 - Standard Test Methods for Evaluating the Degree of Chalking of Exterior Paint Films.

Specifier's note:    Article below includes submittal of relevant data to be furnished by Contractor before, during and after construction.    Coordinate this Article with Architect's    and Contractor's duties and responsibilities in Conditions of the Contract and Division 1 Submittal Procedures Section.

1.04	SUBMITTALS

A.	Product Data:    Sun shade manufacturer's data sheets on each product to be used, including:

1.	Finish manufacturer's data sheet showing physical and performance characteristics.

2.	Storage and handling requirements and recommendations.

3.	Installation instructions and recommendations.

4.	Specimen warranty for finish, as specified herein.

5.	Maintenance instructions.

B.	Shop Drawings: Show layout and elevations, profiles, styles, part numbers, dimensions and thickness of materials, finishes, connections, details and location of joints, method of anchorage, type of anchors and backing supports.

1.	Differentiate between shop fabrication and field installation.

2.	Indicate substrates and adjacent work specified in related sections with which the exterior sun shades must be coordinated.

C.	Samples:    Submit samples of each component, and fasteners to be utilized in Sun shade assembly    with appropriate finish.

D.	Structural calculations provided upon request.    Provide engineering calculations for the sun shade system and mounting brackets, prepared by an engineer registered in the state the project is located.

E.	Warranty:    Provide written warranty that sun shade products will be free of defective materials or workmanship for a period of one year from date of    installation.

F.	Certifications:    Manufacturer's certification that sun shade meets design criteria, Drawings and specification requirements.

Specifier's note:    Article below to include qualifications, prerequisites, standards, limitations, and criteria to establish the requirements for the level of quality for products and workmanship for the work of this section.    Coordinate Article with Division 1 Quality Assurance Section.

1.05	QUALITY ASSURANCE

A.	Field Measurements:    Verify actual dimensions by field measurement before fabrication; show recorded measurements on shop drawings.

B.	Exterior sunshade Manufacturer Qualifications:    Company specializing in manufacturing products specified in this section with continuous experience of over [3] [__][CRL to advise, and validate years of experience] years.

C.	Installer Qualifications:    Experienced in performing work of the type specified in this section.

1.	With minimum 3 years of documented experience in installation of exterior sun shades

system    similar to the work of this section.

2.	Approved by sun shade manufacturer.

D.	Design Engineer's Qualifications:    When required by building authority having jurisdiction, Design structural supports and anchorages under direct supervision of a Structural Engineer experienced in design of this type of Work and licensed in [insert    job specific state, or location].

E.	For quality and delivery control, sunshades must be purchased from a single source.      Sub contracting of shade assembly is not acceptable.

Specifier's note:    Retain paragraph below if mock-up assemblies; erected either on-site, or off-site, are required.    Mock-ups can be used to establish standards of quality for workmanship, review of construction sequence/operation, and coordination of work of related sections.    Coordinate with Division 1 Quality Control Section; Mock-up requirements.

F.	Mock-Up:    Provide a mock-up for evaluation of fabrication workmanship.

1.	Locate on project site [at remote location selected by Owner] as indicated on Drawings.

2.	Provide units finished as specified.

3.	Mock-up may remain as part of the Work.

1.06	DELIVERY, STORAGE, AND HANDLING

A.	Deliver products in manufacturer's original, unopened, undamaged containers with identification labels intact.

1.	Protect finishes by applying heavy duty removable plastic film during production.

2.	Package for protection against transportation damage.

3.	Provide markings to identify components consistently with drawings.

4.	Exercise care in unloading, storing and installing panels to prevent bending, warping, twisting and surface damage.

B.	Store products protected from exposure to harmful weather conditions and at temperature conditions recommended by manufacturer.

1.07	WARRANTY

A.	See Section 01 78 00 - Closeout Submittals, for additional warranty requirements.

B.	See manufacturer for additional information on extended warranty periods for high performance coatings for aluminum sun shades.

PART 2    PRODUCTS

2.01	MANUFACTURERS

A.	C.R. Laurence Co, Inc., P.O. Box 58923, Los Angeles, CA 90058-0923; Tel: (800) 421-6144 or (323) 588-1281 Ext. 7770; Fax: (866) 921-0532 or (323) 584-5226; Email: archmetals@crlaurence.com.

B.	Substitutions:    No substitutions are permitted.

2.02	EXTERIOR SUN SHADE SYSTEM

A.	 Basis of Design:    _C.R. Laurence 7700 Series______Aluminum sun shades.    Factory fabricated, pre-finished aluminum units; 36 inch projection; fixed bar style with a tapered bull nose edge.    Units are shipped pre-assembled to site location.    Standard length; 72 inch, 96 inch, and 120 inch.    Custom lengths are available upon request.

B.	Components:

1.	Outriggers: 3/8 inch thick aluminum plate; ASTM B 209, 5052-H32 alloy.

2.	Fascia: 1/8 inch thick aluminum extrusion; ASTM B 221, 6063-T5 alloy, with bullnose edge.

3.	Blades: 1/8 inch thick aluminum bars; ASTM B 209, 6061-T6 alloy;    spaced 2 inches on center. Blades are welded to cross bars, and edge frame for a complete welded assembly.

4.	Back plate: 1/2 inch thick by 8 inch wide aluminum bar; ASTM B 209, 6061-T6 alloy; continuous for full length of unit for mounting to structure.

5.	Cover plate:    1/16 inch thick; ASTM B 209, 6061-T6 to conceal mounting hardware.

6.	Anchors and Inserts: Use non-Ferrous metal or hot dip galvanized anchors and inserts for installation and elsewhere as required for corrosion resistance. Use stainless steel or lead expansion bolt devices for drill-in place anchors. Furnish inserts, as required, to be set into concrete or masonry work.

7.	Fasteners: Fasteners shall be stainless steel bolts, studs, sheet metal screws, and other types of size and spacing as recommended by manufacturer for specific installation conditions and as detailed on approved shop drawings.

2.03	FABRICATION

A.	Provide fixed sun shades and accessories of design, material, sizes, depth, arrangement, and thickness as indicated on Drawings and as required for optimal performance with respect to strength; durability; and uniform appearance.

B.	Sun shade components are welded, assembled and finished at factory. Units are packaged and shipped to site ready for erection.

C.	Corners: Miter sunshade units at corners as indicated on shop drawings.

D.	Fabrication tolerances:    Maximum allowable tolerances.

1.	Bow: + - 1/4 inch.

2.	Dimensional width or length: + - 1/8 inch.

3.	Squareness: + - 1/8 inch.

2.04	FINISHES

A.	Standard finish is powder coat: Metallic Silver. [CRL to provide quality of finish (ie, impact resistance, hardness, salt-spray resistance, weatherability, gloss retention, fade resistance, etc.)].

Specifier's note:    Paragraph above is CRL standard finish.    Paragraphs below are optional finishes available from CRL. Select desired finish and delete finishes not used.

B.	High performance powder coating:    Spraylat Newlar thermosetting powder coating; exceeds the performance requirements of AAMA 2605.    Corrosion protection:    Meets 4000 hr. salt spray when tested in accordance with ASTM B117.

C.	High Performance PVDF coating:    Duranar contains 70% Kynar 500 or Hylar 5000 fluoropolymer resin; exceeds the performance requirements of AAMA 2605.

PART 3    EXECUTION

3.01	EXAMINATION

A.	Examine substrate conditions in areas to receive the work.    Verify dimensions, tolerances, and interface with adjacent work.    Do not proceed until any unsatisfactory conditions have been corrected.

3.02	PREPARATION

A.	Prior to fabrication, field verify required dimensions.

B.	Coordinate sun shade installation with provision of [exterior wall system] [window framing system] [curtain wall system] to ensure proper structural support is provided, attachment of sun shades is compatible with substrate, and weathertightness of exterior envelop is maintained.

3.03	INSTALLATION

A.	Install in accordance with manufacturer's written instructions and recommendations for installation of the work.

B.	Verify dimensions of supporting structure at the site by accurate field measurements so that the work will be accurately designed, fabricated and fitted to the structure.

C.	Anchor Sunshade to building substructure as indicated on drawings, shop drawings and as specified.

D.	Separate dissimilar metals; use gasket fasteners, isolation shims, or isolation tape where needed to eliminate possibility of electrolytic action between metals.

E.	Do not erect warped, bowed, deformed or otherwise damaged or defaced members.    Remove and replace any members damaged in the erection process as directed.

F.	Set units level, plumb and true to line, with uniform joints.

G.	 Outrigger installation:

1.	 Verify centerline dimensions.

2.	 Install the two outer most outriggers plumb and level to the substrate.

3.	    Then, shoot a line from outrigger to outrigger to find the depth dimension of the outer line.

4.	Proceed with the installation by attaching the middle outrigger, shimming as required.

5.	Shims:      /see pages M284 M285 for shims. Note verify again all centerline dimensions.

H.	Erection Tolerances:

1.	Variation from level: [+/- _____"] maximum in 20'-0" runs, non-cumulative.

2.	Maximum Offset From True Alignment Between Adjacent Members Butting or In Line: [1/16] inch.

3.	Do not field cut or trim units.    Cut and trim component parts during erection only with the approval of the manufacturer or fabricator, and in accordance with his recommendations. Restore finish completely. Remove and replace members where cutting and trimming has impaired the strength or appearance of 	the assembly as directed.

3.04	CLEANING

A.	Remove site cuttings from finish surfaces.

B.	Clean and wash prefinished surfaces with mild soap and water; rinse with clean water.

C.	Clean aluminum surfaces in accordance with recommendations found in AAMA 609 and 610. Do not use agressive alkaline, TSP, acid cleaners, or abrasive cleaners    on aluminum surfaces.

D.	Clean stainless steel surfaces with non-abrasive detergents, soap, ammonia and warm water; rinse with clean water.

END OF SECTION