ALUMINUM VERTICAL SLIDING SERVICE WINDOW

SECTION 08580

# PART 1 – GENERAL

1.01 SUMMARY

- This section includes:

- Aluminum, medium-duty commercial vertical sliding service windows as indicated in drawings and in sections.

1.02 SUBMITTALS

- Product Data: Submit Manufacturer’s technical product data substantiating that products comply.

- Shop drawings: Submit for fabrication and installation of windows. Include details, elevations and installation requirement of finish hardware and cleaning.

C.	Certification: Provide printed data in sufficient detail to indicate compliance with the contract documents.

1.03 DELIVERY, STORAGE, AND HANDLING

- Deliver windows crated to provide protection during transit and job storage

- Inspect windows upon delivery for damage. Unless minor defects can be made to meet the Architect’s specifications and satisfaction, damaged parts should be removed and replaced.

- Store windows at building site under cover in dry location.

1.04 PROJECT CONDITIONS

- Field measurements: Check opening by accurate field measurement before fabrication. Show recorded measurements on shop drawings. Coordinate fabrication schedule with construction progress to avoid delay of work.

1.05 WARRANTY

All material and workmanship shall be warranted against defects for a period of one (1)

year from the original date of purchase.

ALUMINUM VERTICAL SLIDING SERVICE WINDOW

# SECTION 08580 -2

# PART 2 - PRODUCTS

2.01 ACCEPTABLE MANUFACTURER’S

- Basis of design: Design is based on medium-duty commercial aluminum (SW) series, vertical sliding service window manufactured by C.R. Laurence Co., Inc. (800) 421-6144 Part No. V3636

2.02 MATERIALS

- Frames: Aluminum frame modules shall be constructed of 6063-T5 extruded aluminum.  Poly-pile weather stripping and slide locks. Overall frame size to be 36”W x 36”H.

- Finish: All aluminum to be satin anodized.

- Glazing: Units come unglazed. 1/4” vinyl is supplied for glazing selected by architect.

- Screen: Screen included

PART 3 – EXECUTION

3.01 INSTALLATION

- Install window in accordance with manufacturer’s printed instructions and recommendations. Repair damaged units as directed (if approved by the manufacturer and the architect) or replace with new units.

3.02 CLEANING

A.	Clean frame and glazing surfaces after installation, complying with requirements contained in the manufacturer’s instructions. Remove excess glazing sealant compounds, dirt or other substances.

3.03 PROTECTION

- Institute protective measures required throughout the remainder of the construction period to ensure that all the windows do not incur any damage or deterioration, other than normal weathering, at the time of acceptance.

END OF SECTION