STAINLESS STEEL BULLET RESISTANT TRANSACTION WINDOW

SECTION 08580

# PART 1 – GENERAL

1.01 SUMMARY

- This section includes:

- Stainless Steel bullet resistant transaction windows as indicated in drawings and in sections.

- The publication below forms a part of this specification:

1.UNDERWRITERS LABORATORY UL 752 9th Edition

Standard for Bullet Resisting Equipment dated Jan. 27, 1995.

1.02 SUBMITTALS

- Product Data: Submit Manufacturer’s technical product data substantiating that products comply.

- Shop drawings: Submit for fabrication and installation of windows. Include details, elevations and installation requirement of finish hardware and cleaning.

C.	Certification: Provide printed data in sufficient detail to indicate compliance with the contract documents.

1.03 DELIVERY, STORAGE, AND HANDLING

- Deliver windows crated to provide protection during transit and job storage

- Inspect windows upon delivery for damage. Unless minor defects can be made to meet the Architect’s specifications and satisfaction, damaged parts should be removed and replaced.

- Store windows at building site under cover in dry location.

1.04 PROJECT CONDITIONS

- Field measurements: Check opening by accurate field measurement before fabrication. Show recorded measurements on shop drawings. Coordinate fabrication schedule with construction progress to avoid delay of work.

1.05 WARRANTY

All material and workmanship shall be warranted against defects for a period of one (1)

year from the original date of purchase.

STAINLESS STEEL BULLET RESISTANT TRANSACTION WINDOW

# SECTION 08580 -2

# PART 2 - PRODUCTS

2.01 ACCEPTABLE MANUFACTURER’S

- Basis of design: Design is based on Stainless Steel bullet resistant transaction windows, # S11W12S & S11W18S manufactured by C.R. Laurence Co., Inc. (800) 421-6144

2.02 MATERIALS

- Frames: Stainless Steel bullet resistant frame modules shall be to the standards established by U.L. 752 Protection Level _____(specify level 1,2,or3).  Frames are to be constructed of 11 ga. stainless steel # 4 finish lined with U.L. listed bullet resistant fiberglass for levels 2 and 3. All joints are mitered and welded. All exposed welds shall be ground flush and finished smooth.  Replacement of glazing shall be from the secure side of the window or wall unit and does not require the removal of the frame from the opening. Shapes and sizes are to be in accordance with the contract drawings. FRAMES MUST UTILIZE TESTING RECOGNIZED UNDER THE STANDARDS ESTABLISHED BY U.L. 752 FOR BULLET RESISTANT COMPONENTS.

- Finish: # 4 stainless steel

- Glazing: The glazing must be in accordance with U.L. 752 testing standards

Level ____ (specify level 1, 2, or 3). Laminated glass, polycarbonate, or acrylic (specify type of glazing material desired).

- Shelf: Provide a shelf not less than 2” thick with recessed deal tray. The shelf is to be the full width of the window and a minimum of 12” deep centered under the glazing. (Options include: 18” deep shelf, 16 ga. stainless steel No. 4 finish or high pressure laminate shelf, drain holes, and weatherflap.)

- Surround Sound: For interior windows only. Provide for two way “natural voice” or “surround sound” communication permitted by the design of the window jambs and glazing technique. Units must be manufactured in strict accordance with the specifications, design and details. No field alterations to the construction of the units fabricated under the acceptable standards shall be allowed unless approved by the manufacturer and the architect

STAINLESS STEEL BULLET RESISTANT TRANSACTION WINDOW

SECTION 08580 -3

PART 3 – EXECUTION

3.01 INSTALLATION

- Install frames and glazing in accordance with manufacturer’s printed instructions and recommendations. Repair damaged units as directed (if approved by the manufacturer and the architect) or replace with new units.

3.02 CLEANING

A.	Clean frame and glazing surfaces after installation, complying with requirements contained in the manufacturer’s instructions. Remove excess glazing sealant compounds, dirt or other substances.

3.03 PROTECTION

A.	Institute protective measures required throughout the remainder of the construction period to ensure that all the windows do not incur any damage or deterioration, other than normal weathering, at the time of acceptance.

## END OF SECTION