ALL GLASS ENTRANCES

SECTION 08490-1

PART 1 - GENERAL

1.01 	SUMMARY

A.	This section includes:

1.	Vestibule doors matching entrance doors.

2.	Sidelites.

B.	Related work in other sections:

1. 	Lock cylinders for tempered glass sliding entrance doors are specified in Division 8

section “Finished Hardware”.

1.02	SUBMITTALS

A.	Product Data: Submit Manufacturer’s product data for all glass entrance systems

including:

1. 	Manufacturer’s standard details and fabrication method.

2.	Data on finishing, hardware and accessories.

3.	Recommendations for maintenance and cleaning of exterior finish surfaces.

4.	Test data on fabricated door.

B.	Shop drawings for each all glass entrance system are required, including:

1.	Layout and installation details.

2.	Elevations at 1/4-inch scale.

3.	Detail sections of fittings.

4.	Hardware mounting heights.

5.	Anchorage and reinforcement.

6. 	Glazing details.

ALL GLASS ENTRANCES

SECTION 08490-2

C. 	Samples for approval:

1.	Submit pairs of samples of each specified metal color and finish on 9-inch long

sections of extrusions or formed shapes.

2.	Submit samples of glass approximately 6 inches square showing the edge

conditions.

1.03	QUALITY ASSURANCE

A.	Installer qualifications: Engage an experienced installer who has completed installations of all glass sliding entrances similar in design and extent to those required for the project and whose work has resulted in construction with a record of successful in service performance.

B.	Manufacturer’s qualifications: Provide all glass sliding entrances produced by a firm 		experienced in manufacturing entrance systems that are similar to those indicated for this 	project and that have a record of successful in service performance. All door rail systems 	must be tested.

C.	Single source responsibility:	Obtain all glass sliding entrance systems from a single 		manufacturer, to ensure full compatibility and warranty of parts.

D.	Design criteria: The drawings indicate the size, profile and dimensional requirements of the all glass sliding entrance system required and are based on the specific types and models indicated. All glass sliding entrances by other manufacturers may be considered, provided deviations in dimensions and profiles are minor and do not change the design concept as judged by the Architect. The burden of proof of equality is on the proposer.

E.	Safety glass standard: Provide tempered glass components that comply with ANSI Z97.1

and testing requirements of CPSC 16 CFR 1201 Category II.

F.	Testing criteria for sliding door rail: The door rail should also be subject to a temperature pull-off test at temperatures from –10 degrees F. to 150 degrees F.  The rail shall remain stationary throughout this test while a ¼ ton pressure is applied.

ALL GLASS ENTRANCES

SECTION 08490 -3

1.04	DELIVERY, STORAGE, AND HANDLING

A. 	Deliver all glass sliding entrances and related components in the manufacturer’s original 	protective packaging. Do not deliver entrance units until the work is ready for their 	installation.

1.	Inspect components for damage upon delivery. Unless minor defects in metal 			components can be made to meet the Architect’s specifications and satisfaction, 			damaged parts should be removed and replaced.

1.05	PROJECT CONDITIONS

A. 	Field measurements: Check opening by accurate field measurement before fabrication.

Show recorder measurements on shop drawings. Coordinate fabrication schedule with 	construction progress to avoid delay of the work and possible damage to the finished 	product.

1.	Where necessary, proceed with fabrication without measurement and coordinate 			fabrication tolerances to insure proper fit.

PART 2 - PRODUCTS

2.01	ACCEPTABLE MANUFACTURER’S

A. 	Basis of design: Design is based on an all glass sliding entrance door system featuring heavy tempered glass secured to a CRL Wedge-Lock Heavy Glass Sliding Door System, manufactured by: C.R. Laurence Co., Inc. 800-421-6144.  (Specify Bottom Rolling or Overhead Track Sliding.)

B.	Subject to compliance with requirements, all glass entrances from other manufacturers

meeting the specified requirements may be acceptable.

2.02	MATERIALS

A.	Glass: Provide flat, fully tempered glass in thickness indicated for doors and sidelites.

Comply with requirements of ASTM C 1048 for FT (fully tempered), Conditions A 	(uncoated surfaces), Type 1 (transparent) Class 1 (clear) glass.

1.	Thickness: ½ inch or 3/8 inch (3/8 inch for interior and subject to dimension 			restrictions).

ALL GLASS ENTRANCES

SECTION 08490 -4

2.	Edge treatment: Provide machine ground and polished edges for exposed glass

edges of doors and sidelites and flat ground edges for butting glass edges.

3.	Manufacturers:_____________________________________.

B.	Door fittings: Provide CRL Wedge-Lock Door Rails in required profile, size and glass 	thickness (Tapered or Square in 4"or 6" square heights) as selected by the Architect. 	Comply with requirements indicated for kind and form of metal finish.

1.	Aluminum: Provide fittings fabricated from aluminum extrusions of alloy and 			temper recommended by manufacturer for use intended and required for 				application of finish indicated, but not less than strength and durability properties 			specified in ASTM B 221 for 6063-T5.

C.	Accessory fittings: Provide manufacturer’s standard accessory fittings of the type

indicated. Comply with requirements indicated for kind and form of metal and finish of

door fittings.

1.	Sidelite systems: Provide sidelite systems matching metal and finish of door 			fittings. The design of head and sill details is based upon C.R. Laurence Co. Inc.

Top-Load Gasket Sidelite Rails (Tapered or Square in 4" or 6" square heights) as

selected by the Architect. Top-Load gasket, is cord reinforced EPDM Rubber for 			long life and no stretching.

D.	Anchors and fasteners: Manufacturer’s standard concealed anchors and fastenings. Do

not use exposed fasteners.

2.03	HARDWARE

A.	General: Provide heavy-duty hardware units as indicated, scheduled or required for

operation of each type of door, including the following items of sizes, numbers and type

recommended by the manufacturer for the type of service required. Provide metal and

finish for exposed parts to match the finish of the door rails.

B.	Pull Set: Provide handles selected by the Architect and supplied by C.R. Laurence.

ALL GLASS ENTRANCES

SECTION 08490- 5

C.	Locks: Equip exterior doors with manufacturer’s locksets that accept a standard cylinder

with related components. Comply with the following:

1.	Location and function: Provide round throw deadbolt in continuous bottom fitting.

Lock to be operated by key outside and thumbturn inside.

D.	Threshold: Provide manufacturer’s standard extruded aluminum threshold in mill finish.

Coordinate cutouts with operating hardware. Include anchors.

2.04	FABRICATION

A. 	General: Fabricate all glass sliding entrance components to designs and sizes indicated. Sizes of door and profile requirements of fittings and hardware are indicated on the drawings.

1.	Locate and provide holes and cutouts in glass to receive hardware before 				tempering glass. Do not permit cutting, drilling or other alterations to glass after 			tempering.

2.	Fabricate work to accommodate required fittings, hardware, anchors,

reinforcement, and accessory items.

B.	Prefabrication: Complete fabrication, assembly, finishing, hardware application and other

work to the greatest extent possible before shipment to the project site. Disassemble

components only as necessary for shipment and installation.

C.	Continuity: Maintain accurate relation of planes and angles with hairline fit of 		contracting members.

2.05	METAL FINISHES: (List one or more finishes)

A. 	US-28 Clear Satin Anodized Aluminum

313 Dark Bronze Anodized

US-32 Polished Stainless Steel Clad

US-32D Brushed Stainless Steel Clad

US-3 Polished Brass Clad

PVD “Perma Brass” Polished Clad

Other Custom Metal and Painted finishes available

B.	Preferable use of PVD “Perma Brass” over US-3 Polished Brass Cladding to reduce

maintenance cost in the future.

ALL GLASS ENTRANCES

SECTION 08490 -6

PART 3 - EXECUTION

  - EXAMINATION

A.	Examine substrates and supports with the installer, present for compliance with

requirements indicated, installation tolerances and other conditions that affect the

installation of all glass entrances and storefronts. Correct unsatisfactory conditions before

proceeding with the installation.

1.	Do not proceed with installation until unsatisfactory conditions are corrected.

3.02	INSTALLATION

A.	Install all glass sliding entrance door and associated components in accordance with

manufacturer’s printed instructions and recommendations.

1.	Verify units level, plumb and true line.

2.	Lubricate hardware and other moving parts.

3.03	ADJUSTING

A.	Adjust doors and hardware to provide a tight fit at meeting points. Doors to be aligned                 without removal.

B.	Hardware: Adjust operating hardware to ensure proper operation. Coordinate cylinder installation.

3.04	CLEANING

A.	Clean door and frame surfaces after installation, exercising care to avoid damage to the 	finish.

B.	Clean glass surfaces after installation, complying with requirements contained in the “Glass and Glazing” section for cleaning and maintenance. Remove excess glazing sealant compounds, dirt or other substances.

3.05	PROTECTION

A.	Institute protective measures required throughout the remainder of the construction 	period to ensure that the all glass entrances do not incur any damage or deterioration,

other than normal weathering, at the time of acceptance.

END OF SECTION