ELECTRIC FULLY AUTOMATIC DELUXE SLIDING SERVICE

WINDOW

SECTION 08580

# PART 1 – GENERAL

1.01 SUMMARY

- This section includes:

- Electric, fully automatic deluxe sliding service windows as indicated in drawings and in sections.

1.02 SUBMITTALS

- Product Data: Submit Manufacturer’s technical product data substantiating that products comply.

- Shop drawings: Submit for fabrication and installation of windows. Include details, elevations and installation requirement of finish hardware and cleaning.

C.	Certification: Provide printed data in sufficient detail to indicate compliance with the contract documents.

1.03 DELIVERY, STORAGE, AND HANDLING

- Deliver windows crated to provide protection during transit and job storage

- Inspect windows upon delivery for damage. Unless minor defects can be made to meet the Architect’s specifications and satisfaction, damaged parts should be removed and replaced.

- Store windows at building site under cover in dry location.

1.04 PROJECT CONDITIONS

- Field measurements: Check opening by accurate field measurement before fabrication. Show recorded measurements on shop drawings. Coordinate fabrication schedule with construction progress to avoid delay of work.

1.05 WARRANTY

All material and workmanship shall be warranted against defects for a period of one (1)

year from the original date of purchase.

ELECTRIC FULLY AUTOMATIC DELUXE SLIDING SERVICE

WINDOW

# SECTION 08580 -2

# PART 2 - PRODUCTS

2.01 ACCEPTABLE MANUFACTURER’S

- Basis of design: Design is based on aluminum (EDW) series,  Electric Fully Automatic Deluxe Sliding Service Window manufactured by C.R. Laurence Co., Inc. (800) 421-6144

2.02 MATERIALS

- Frames: 4-1/2” Aluminum frame modules shall be constructed of 6063-T5 extruded aluminum. Replacement and servicing of glass shall be from the clerk side of the window by means of an access panel in the top header and does not require the removal of the frame from the opening. Window glides on top-hung heavy-duty ball bearing slides. Poly-pile weather stripping. With thumbturn lock and locked/unlocked indicator. Infrared sensing device and an on/off/manual switch enclosed in a stainless steel housing. Overall frame sizes are to be in accordance with the contract drawings. Safety features include 24V DC UL/CSA Approved Class 2 Transformer, Auto Reverse if window is blocked, “Soft Close” feature in “Pinch Zone” and a Mechanical Positive Slip Clutch.

- Finish: All aluminum to be clear anodized, duranodic bronze, powder or Kynar painted (specify color).

- Glazing: The glazing is ¼” to ½” in thickness. Options include tempered, wire, laminated, tinted, and insulating glass (specify type of glazing material desired).

- Options: Stainless steel shelf with control panel enclosed, stainless steel sill with control panel enclosed, aluminum half bottom track, aluminum full bottom track, both with separate control panel assembly.  (specify desired options).

- Available in the following configurations: OX, XO, and OXXO. (Specify desired configuration.)

PART 3 – EXECUTION

3.01 INSTALLATION

- Install window in accordance with manufacturer’s printed instructions and recommendations. Repair damaged units as directed (if approved by the manufacturer and the architect) or replace with new units.

3.02 CLEANING

A.	Clean frame and glazing surfaces after installation, complying with requirements contained in the manufacturer’s instructions. Remove excess glazing sealant compounds, dirt or other substances.

ELECTRIC FULLY AUTOMATIC DELUXE SLIDING SERVICE

WINDOW

# SECTION 08580 -3

3.03 PROTECTION

A.	Institute protective measures required throughout the remainder of the construction period to ensure that all the windows do not incur any damage or deterioration, other than normal weathering, at the time of acceptance.

## END OF SECTION