Mirro-Mastic®

Directions for Substrate and Mirror Preparation

- Do not use on a painted surface, wall covering, or treated wood. - For best adhesion, the mirror, substrate, and mastic should be at room temperature (72ºF, 22ºC). - Make sure mirror and substrate are free of dust, clean, and dry. On nonporous substrates, such as glass, tile, or metal, sealing is not necessary. On porous substrates, such as drywall or wood, use Mirro-Mastic® Bond (or a primer or sealer, not paint) on the substrates and allow it to dry. Painted surfaces should be sanded through to the original surface and the substrate cleaned and sealed where the mastic is to be applied. Wall coverings should be removed and the substrate cleaned and sealed where the mastic is to be applied. - Support mirror at the bottom. Mechanical fasteners should be used for all mirror applications.

Directions for use of Mirro-Mastic®

- Apply Mirro-Mastic® to the mirror or substrate in a minimum of one (1) ping-pong ball size mound for every square foot (929 square centimeters) of mirror . Do not apply mastic too close to the edge to prevent "squeeze out". Place the mounds so space will be left between them when the mirror is installed. - Press mirror firmly in place making good contact between the mirror, mastic, and substrate. Mastic should spread to a pat approximately 4-1/2 inches (114 millimeters) in diameter. The mastic needs air circulation to cure properly. Curing time will depend on temperature, humidity, type of substrate, and amount of air that can reach the mastic. With minimum coverage, one gallon of Mirro-Mastic® will install approximately 130 square feet (12 square meters) of mirror, one 29 oz. cartridge will install approximately 30 square feet (2.8 square meters) of mirror, and one 11 oz. cartridge will install approximately 12 square feet (1.1 square meters) of mirror. When using a gallon or five-gallon drum, the 1234PA Electric Applicator and MS97 Mastic Stik are available for ease of application. - Any clean-up can be done with mineral spirits. For ceiling applications or for technical questions not covered in our directions, please contact CRL Technical Sales (800) 421-6144, Ext. 777 or by e-mail at techsales@crlaurence.com.

WARRANTY DISCLAIMER
Nothing herein shall be deemed to constitute a warranty, either express or implied, by Palmer Products Corporation with respect to Mirro-Mastic®  or any other Palmer product. Palmer hereby disclaims all warranties, including without limitation, implied warranties of merchantability and fitness for a particular purpose. This disclaimer shall extend to all users of Palmer products and to any third party.  Any dispute or claim of whatsoever type arising out of the use of any Palmer product shall be governed by the laws of the Commonwealth of Kentucky. Palmer's sole liability, if any, and any user's or third party claimant's sole and exclusive remedy relating to any such dispute or claim shall be replacement of the Palmer product used.