ALL GLASS ENTRANCES

SECTION 08450

PART 1 - GENERAL

1.01 	SUMMARY

A.	This section includes:

1.	Vestibule doors matching entrance doors.

2.	Sidelites.

B.	Related work in other sections:

1. 	Lock cylinders for tempered glass entrance doors are specified in Division 8

section “Finished Hardware”.

1.02	SUBMITTALS

A.	Product Data: Submit Manufacturer’s product data for all glass entrance systems

including:

1. 	Manufacturer’s standard details and fabrication method.

2.	Data on finishing, hardware and accessories.

3.	Recommendations for maintenance and cleaning of exterior finish surfaces.

4.	Test data on fabricated door.

B.	Shop drawings for each all glass entrance system are required, including:

1.	Layout and installation details.

2.	Elevations at 1/4-inch scale.

3.	Detail sections of fittings.

4.	Hardware mounting heights.

5.	Anchorage and reinforcement.

6. 	Glazing details.

ALL GLASS ENTRANCES

SECTION 08450

C. 	Samples for approval:

1.	Submit pairs of samples of each specified metal color and finish on 9-inch long

sections of extrusions or formed shapes.

2.	Submit samples of glass approximately 6 inches square showing the edge

conditions.

1.03	QUALITY ASSURANCE

A.	Installer qualifications: Engage an experienced installer who has completed installations 	of all glass entrances similar in design and extent to those required for the project and 	whose work has resulted in construction with a record of successful in service 		performance.

B.	Manufacturer’s qualifications: Provide all glass entrances produced by a firm 		experienced in manufacturing entrance systems that are similar to those indicated for this 	project and that have a record of successful in service performance. All door rail systems 	must be tested.

C.	Single source responsibility:	Obtain all glass entrance systems from a single 		manufacturer, to ensure full compatibilty and warranty of parts.

D.	Design criteria: The drawings indicate the size, profile and dimensional requirements of 	the all glass entrance system required and are based on the specific types and models

indicated. All glass entrances by other manufacturers may be considered, provided

deviations in dimensions and profiles are minor and do not change the design concept as 	judged by the Architect. The burden of proof of equality is on the proposer.

E.	Safety glass standard: Provide tempered glass components that comply with ANSI Z97.1

and testing requirements of CPSC 16 CFR 1201 Category II.

F.	Testing criteria for door rail: Preliminary testing conducted in house produced the

following results:

1. 	1,250,000 cycles with no apparent wear or malfunction (testing is ongoing).

2.	Our 9" Corner Patch Door Rail was subjected to 980 lbs. of pull for 30 continuous 		days with no movement or slippage detected.

Currently, an independent engineering firm is testing our door rail for 1,000,000 		openings, and a temperature pull-off test at 1 ton load from -10 degrees F. to 150 	degrees F. As soon as the test are completed, results will be made available and our

test criteria will be updated to reflect the results.

ALL GLASS ENTRANCES

SECTION 08450 - 2

1.04	DELIVERY, STORAGE, AND HANDLING

A. 	Deliver all glass entrances and related components in the manufacturer’s original 	protective packaging. Do not deliver entrance units until the work is ready for their 	installation.

1.	Inspect components for damage upon delivery. Unless minor defects in metal 		components can be made to meet the Architect’s specifications and satisfaction, 		damaged parts should be removed and replaced.

1.05	PROJECT CONDITIONS

A. 	Field measurements: Check opening by accurate field measurement before fabrication.

Show recorder measurements on shop drawings. Coordinate fabrication schedule with 	construction progress to avoid delay of the work and possible damage to the finished 	product.

1.	Where necessary, proceed with fabrication without measurement and coordinate 		fabrication tolerances to insure proper fit.

ALL GLASS ENTRANCES

SECTION 08450 -3

PART 2 - PRODUCTS

2.01	ACCEPTABLE MANUFACTURER’S

A. 	Basis of design: Design is based on an all glass entrance door system featuring heavy 	tempered glass secured to a CRL Wedge-Lock Door Rail System, manufactured by

C.R. Laurence Co., Inc.

B.	Subject to compliance with requirements, all glass entrances from other manufacturer’s

meeting the specified requirements may be acceptable.

2.02	MATERIALS

A.	Glass: Provide flat, fully tempered glass in thickness indicated for doors and sidelites.

Comply with requirements of ASTM C 1048 for FT (fully tempered), Conditions A 	(uncoated surfaces), Type 1 (transparent) Class 1 (clear) glass.

1.	Thickness: ½ inch or 3/8 inch (3/8 inch for interior and subject to dimension 		restrictions).

2.	Edge treatment: Provide machine ground and polished edges for exposed glass

edges of doors and sidelites and flat ground edges for butting glass edges.

3.	Manufacturers:_____________________________________.

B.	Door fittings: Provide CRL Wedge-Lock Door Rails in required profile, size and glass 	thickness (Tapered or Square in 4", 6" or 10" heights) as selected by the Architect. 	Comply with requirements indicated for kind and form of metal finish.

1.	Aluminum: Provide fittings fabricated from aluminum extrusions of alloy and 		temper recommended by manufacturer for use intended and required for 			application of finish indicated, but not less than strength and durability properties 		specified in ASTM B 221 for 6063-T5.

C.	Accessory fittings: Provide manufacturer’s standard accessory fittings of the type

indicated. Comply with requirements indicated for kind and form of metal and finish of

door fittings.

1.	Overhead door stop: Provide overhead door stop systems.

2.	Sidelite systems: Provide sidelite systems matching metal and finish of door 		fittings. The design of head and sill details is based upon C.R. Laurence Co. Inc.

Top-Load Gasket Sidelite Rails (Tapered or Square in 4" or 6" heights) as

selected by the Architect. Top-Load gasket, is cord reinforced EPDM Rubber for 		long life and no stretching.

# ALL GLASS ENTRANCES

SECTION 08450 - 4

D.	Anchors and fasteners: Manufacturer’s standard concealed anchors and fastenings. Do

not use exposed fasteners.

E.	Weather-stripping: Manufacturer’s standard sweep-type weather-stripping on exterior 	doors.

2.03	HARDWARE

A.	General: Provide heavy duty hardware units as indicated, scheduled or required for

operation of each type of door, including the following items of sizes, numbers and type

recommended by the manufacturer for the type of service required. Provide metal and

finish for exposed parts to match the finish of the door rails.

B.	CRL by C.R. Laurence Co., 9100 Series Medium Duty long spindle Overhead 		Concealed Door Closers and 9200 Series Heavy Duty long spindle Overhead Concealed 	Door Closers are double acting and fit into CRL 4-1/2" x 1-3/4" Single or Double Closer 	Headers. Closers secure to the center hung arm in the top door rail. Provide top 		adjustable arm, bottom adjustable pivot and finished cover plate if required. Comply with 	manufacturer’s recommendations for closer size, depending upon door size, exposure to 	weather, and anticipated frequency of use.

C.	CRL by C.R. Laurence Co., 8300 Series and 8400 Series Medium Duty Floor Door 	Closers are double acting and supplied with cement box and Brushed S/S cover plate. 	Cover plates in other finishes are available.  Provide bottom adjustable arm, top 		adjustable pivot and finished cover plate if required. Comply with manufacturer’s 	recommendations for closer size, depending upon door size, exposure to weather, and 	anticipated frequency of use.

Include the following:

1.	Non-hold-open closers must comply with ADA Handicap requirements.

2.	Consult manufacturer for closer recommendations on doors over 264 lbs.

D.	Push-Pull Set: Provide handles selected by the Architect and supplied by C.R. Laurence.

E.	Locks: Equip exterior doors with manufacturer’s locksets that accept a standard cylinder 	with related components. Comply with the following:

1.	Location and function: Provide round throw deadbolt in continous bottom fitting.

Lock to be operated by key outside and thumbturn inside with end load capability.

F.	Cylinders or Magnetic Locks are supplied as described under  Division 8 section, for 	keying into building system.

ALL GLASS ENTRANCES

SECTION 08450 - 5

G.	Threshold: Provide manufacturer’s standard extruded aluminum threshold in mill finish.

Coordinate cutouts with operating hardware. Include anchors and jamb clips.

2.04	FABRICATION

A. 	General: Fabricate all glass entrance components to designs and sizes indicated. Sizes of

door and profile requirements of fittings and hardware are indicated on the drawings.

1.	Locate and provide holes and cutouts in glass to receive hardware before 			tempering glass. Do not permit cutting, drilling or other alterations to glass after 		tempering.

2.	Fabricate work to accomodate required fittings, hardware, anchors,

reinforcement, and accessory items.

B.	Prefabrication: Complete fabrication, assembly, finishing, hardware application and other

work to the greatest extent possible before shipment to the project site. Disassemble

components only as necessary for shipment and installation.

C.	Continuity: Maintain accurate relation of planes and angles with hairline fit of 		contracting members.

2.05	METAL FINISHES: (List one or more finishes)

A. 	US-28 Clear Satin Anodized Aluminum

313 Dark Bronze Anodized

US-32 Polished Stainless Steel Clad

US-32D Brushed Stainless Steel Clad

US-3 Polished Brass Clad

PVD “Perma Brass” Polished Clad

Other Custom Metal and Painted finishes available

B.	Preferable use of PVD “Perma Brass” over US-3 Polished Brass Cladding to reduce

maintenance cost in the future.

ALL GLASS ENTRANCES

SECTION 08450 -6

PART 3 - EXECUTION

3.01	EXAMINATION

A.	Examine substrates and supports with the installer, present for compliance with

requirements indicated, installation tolerances and other conditions that affect the

installation of all glass entrances and storefronts. Correct unsatisfactory conditions before

proceeding with the installation.

1.	Do not proceed with installation until unsatisfactory conditions are corrected.

3.02	INSTALLATION

A.	Install all glass entrance door and associated components in accordance with

manufacturer’s printed instructions and recommendations.

1.	Verify units level, plumb and true line.

2.	Lubricate hardware and other moving parts.

3.03	ADJUSTING

A.	Adjust doors and hardware to provide a tight fit at meeting points and at

weather-stripping for smooth operation and weather tight closure.

B.	Hardware: Adjust operating hardware to ensure proper operation. Set, seal and grout

floor closer cases. Coordinate cylinder installation.

3.04	CLEANING

A.	Clean door and frame surfaces after installation, exercising care to avoid damage to the 	finish.

B.	Clean glass surfaces after installation, complying with requirements contained in the 	“Glass and Glazing” section for cleaning and maintenance. Remove excess glazing 	sealant compounds, dirt or other substances.

3.05	PROTECTION

A.	Institute protective measures required throughout the remainder of the construction 	period to ensure that the all glass entrances do not incur any damage or deterioration,

other than normal weathering, at the time of acceptance.

END OF SECTION