SECTION 05 7310

TAPER-LOC® DRY GLAZE GLASS RAILING SYSTEMS

PART 1 	GENERAL

  - SECTION INCLUDES

- Monolithic Tempered Glass Dry Glazed Railing Assemblies.

  - RELATED SECTIONS

- Section 05 5000 - Metal Fabrications

- Section 05 7000 - Ornamental Handrails & Railings

- Section 05 7300 – Handrails and Railings

- Section 08 8000 - Glazing

  - REFERENCES

- ESR-3269 ICC-ES Evaluation Report, International Code Council Standards for Glass Balustrade Guard Rail Applications

- ASTM C 1048 – Standard Specification for Heat Treated Flat Glass – Kind HS, Kind FT Coated and Uncoated Glass

- NAAMM Metal Finishes Manual; national Association of Architectural Metal Manufacturers

  - SYSTEM DESCRIPTION

- Performance Requirements for Handrail Assembly:

  - Support distributed load of 50 pounds per linear foot (0.73kN/M), applied horizontally at right angles in any direction to the handrail.

  - Support concentrated horizontal load of 200 pounds (0.89kN), applied in any direction at any point along handrail system.

  - 50 lbs (0.22kN) on 1 sf (0.093m2) perpendicular to guard at any location

  - Wind loads 25 psf or as otherwise specified.

  - Distributed loads and concentrated loads not to be applied simultaneously.

  - SUBMITTALS

- Submit under provisions of Section 01 3300.

- Product Data: Submit Manufacturer’s technical product data for railing components and accessories.

- Shop Drawings: Dimensioned drawings of railing assemblies indicating the following:

1.	Elevations; include joint locations, transitions, and terminations.

  - Manufacturer’s installation and maintenance instructions.

- Samples of manufacturer’s finishes (As selected by Architect.)

  - QUALITY ASSURANCE

- Components and installation are to be in accordance with state and local building codes.

- All components and fittings are furnished by the same manufacturer.

  - DELIVERY, STORAGE, AND HANDLING

- Deliver materials properly protected against damage to finished surfaces during transit.

- Inspect materials upon delivery for damage. Unless minor defects can be made to meet the Architect’s specifications and satisfaction, damaged parts should be removed and replaced.

- Store materials at building site under cover in dry location

PART 2 	PRODUCTS

  - MANUFACTURERS

- Acceptable Manufacturer: C.R. Laurence Co., Inc. (CRL)

# Tel: (800) 421-6144 Fax: (800) 587-7501

Email: railings@crlaurence.com

www.crlaurence.com

- Manufacturers of equivalent products will be considered for substitution in accordance with provisions of Section 01 2500 - Product Substitution Procedures.

  - MATERIALS

- Aluminum Components: Conforming to ASTM B 221/ASTM B221M, Alloy 6063- T52

- Stainless Steel Components: Conforming to ASTM A 666, Type 304

- Brass Components: Conforming to ASTM B 248, No. 260, Yellow Brass

  - COMPONENTS

- Glazing: Fully tempered ASTM C 1048 Kind FT, Quality q3. As specified in Section

08 8000

  - Monolithic Tempered Thickness: 1/2 inch (12 mm). (Architect to specify)

  - Monolithic Tempered Thickness: 5/8 inch (15 mm). (Architect to specify)

  - Monolithic Tempered Thickness: 3/4 inch (19 mm). (Architect to specify)

  - Color: Clear, or tint. (Architect to specify)

  - Architect to specify edge type on exposed glass edges. (See section 08 8000.)

- Internal Handrail Cap Connection Sleeves: Metal tube, material compatible with handrail cap material.

- TAPER-LOC® Dry Glazing System: Each TAPER-LOC® Set consists of two Tapers, and one L-Setting Block. Designed for B5A, B5S, B5T, B6S, and B7S Shoe Bases. Patent Pending.

- Shoe Base: (Architect to specify)

  - Profile: CRL Part # B5A; 2-1/2 inches (63.5 mm) wide by 4-1/4 inches (106.4 mm) high rectangular cross-section. Designed to work with CRL's TAPER-LOC® Dry Glazed System with 1/2” (12 mm) monolithic tempered glass.

  - Profile: CRL Part # B5S; 2-1/2 inches (63.5 mm) wide by 4-1/8 inches (104.8 mm) high rectangular cross-section. Designed to work with CRL's TAPER-LOC® Dry Glazed System with 1/2” to 5/8” (12 to 16 mm) monolithic tempered glass.

  - Profile: CRL Part # B5T; 2-1/2 inches (63.5 mm) wide by 4-1/8 inches (104.8 mm) high tapered cross-section. Designed to work with CRL's TAPER-LOC® Dry Glazed System with 1/2” to 5/8” (12 to 16 mm) monolithic tempered glass.

  - Profile: CRL Part # B6S; 2-5/8 inches (63.5 mm) wide by 4-1/8 inches (104.8 mm) high rectangular cross-section. Designed to work with CRL's TAPER-LOC® Dry Glazed System with (15 mm) monolithic tempered glass.

  - Profile: CRL Part # B7S; 2-3/4 inches (69.8 mm) wide by 4-1/8 inches (104.7 mm) high rectangular cross-section. Designed to work with CRL's TAPER-LOC® Dry Glazed System with 3/4” (19 mm) monolithic tempered glass.

  - Material: Aluminum 6063-T52

  - Finish: (Architect to specify.)

    - Base Cladding: Sheet metal cladding added to exposed shoe base sections. Adhere with double-sided tape and/or silicone adhesive. Provide end caps where ends of shoe base sections are exposed.

    - 304 Brushed Stainless (Architect to specify)

    - 304 Polished Stainless (Architect to specify)

    - C260 Polished Brass (Architect to specify)

    - C260 Satin Brass (Architect to specify)

    - 5052 Satin Anodized (Architect to specify)

    - 5052 Dark Bronze Anodized (Architect to specify)

- Metal Cap Railing: (Architect to specify)

  - Profile: Part # GRL10, low profile 11 gauge u-channel 1-5/16 inches (33.3 mm) high.

  - Profile: Part # GRLC10, crisp corner low profile 11 gauge u-channel 1-5/16 inches (33.3 mm) high.

  - Profile: Part # GRUC, u-channel 1-1/2 inches (38.1 mm).

  - Profile: Part # GRCF, crisp corner u-channel 1-1/2 inches (38.1 mm).

  - Profile: Part # GRCR, crisp corner radius top u-channel 1-1/2 inches (38.1 mm).

  - Profile: Part # GR15, round 1-1/2 inches (38.1 mm) diameter.

  - Profile: Part # GRS15, square 1-1/2 inches (38.1 mm).

  - Profile: Part # GRSC15, crisp corner square 1-1/2 inches (38.1 mm).

  - Profile: Part # GR16, round 1.66 inches (42.2 mm) diameter.

  - Profile: Part # GR19, round 1-7/8 inches (48.3 mm) diameter (aluminum only)

  - Profile: Part # GR20, round 2 inches (50.8 mm) diameter.

  - Profile: Part # 337, CRL-Blumcraft round 2 inches (50.8 mm) diameter.

  - Profile: Part # GRS20, square 2 inches (50.8 mm).

  - Profile: Part # GRSC20, crisp corner square 2 inches (50.8 mm).

  - Profile: Part # GR25, round 2-1/2 inches (63.5 mm) diameter.

  - Profile: Part # 338, CRL-Blumcraft round 2-1/2 inches (63.5 mm) diameter.

  - Profile: Part # GRS25, square 2-1/2 inches (63.5 mm).

  - Profile: Part # GR30, round 3 inches (76.2 mm) diameter.

  - Profile: Part # GR35, round 3-1/2 inches (88.9 mm) diameter.

  - Profile: Part # GR40, round 4 inches (101.6 mm) diameter.

  - Profile: Part # GR0V4, oval 4 inches x 2-1/2 inches (101.6 mm x 63.5 mm) (aluminum only)

  - Profile: Part # 324, CRL-Blumcraft square 1-3/8 inches (35 mm).

  - Profile: Part # 339, CRL-Blumcraft rectangular 1-3/4 inches x 3-1/2 inches (44.4 x 90 mm).

  - Profile: Part # 376, CRL-Blumcraft elliptical 2-11/16 inches (68.2 mm).

  - Profile: Part # 398, CRL-Blumcraft rectangular 4 inches (102 mm).

  - Profile: Part # 636, CRL-Blumcraft rectangular 1-3/4 inches x 4 inches (44.5   102 mm).

  - Profile: Part # 637, CRL-Blumcraft rectangular 1-3/4 inches x 5 inches (44.5   127 mm).

  - Profile: Part # 638, CRL-Blumcraft rectangular 1-3/4 inches x 6 inches (44.5   152 mm).

  - Material: _________________.

  - Finish: (Architect to specify.)

- Wood Cap Railing: (Architect to specify)

  - Profile: Part # WCR20, CRL round 2 inches (50.8 mm) diameter.

  - Profile: Part # 346, CRL-Blumcraft round 2 inches (50.8 mm) diameter.

  - Profile: Part # 347, CRL-Blumcraft round 2-1/2 inches (63.5 mm) diameter.

  - Profile: Part # WCR25, CRL round 2-1/2 inches (63.5 mm) diameter.

  - Profile: Part # 397, CRL-Blumcraft 1-3/4 inches x 2-1/8 inches (44.5 x 54 mm)

  - Profile: Part # 372, CRL-Blumcraft 2 inches x 3-3/4 inches (51 x 95.3 mm)

  - Profile: Part # 373, CRL-Blumcraft 4-1/2 inches x 1-5/8 inches (114.3 x 41.3 mm)

  - Profile: Part # 631, CRL-Blumcraft 2-1/2 inches x 4 inches (63.5 x 102 mm)

  - Profile: Part # 632, CRL-Blumcraft 2-1/2 inches x 6 inches (63.5 x 152 mm)

  - Profile: Part # 633, CRL-Blumcraft 2-1/2 inches x 8 inches (63.5 x 203 mm)

  - Profile: Part # WCR30, CRL round 3 inches (76.2 mm) diameter.

  - Wood Type: _________________.

- Handrail Brackets: (Architect to specify)

  - Material: Aluminum

  - Material: Stainless Steel

  - Material: Brass

  - Fabrication: Machined

  - Fabrication: Cast

  - Finish: Match handrail cap finish

- Metal Handrail Tubing: (Architect to specify)

  - Profile: Part # HR15, round 1-1/2 inches (38.1 mm) diameter.

  - Profile: Part # HRH15, heavy-wall round 1-1/2 inches (38.1 mm) diameter

  - Profile: Part # 583, CRL-Bumcraft round 1-1/2 inches (38.1 mm) diameter.

  - Profile: Part # HRS15, square 1-1/2 inches (38.1 mm).

  - Profile: Part # HR19, round 1-7/8 inches (48.3 mm) diameter.

  - Profile: Part # HR20, round 2 inches (50.8 mm) diameter.

  - Profile: Part # 537, round 2 inches (50.8 mm) diameter.

  - Profile: Part # 576, elliptical 2 inches x 1-3/8 inches (50.8 x 35 mm).

  - Profile: Part # PR15, round 1.9 inches (48.3 mm) 1-1/2 inch schedule 40.

  - Profile: Part # PR12, round 1.66 inches (42.2 mm) 1-1/4 inch schedule 40.

  - Profile: Part # PR2, square 2 inches (50.8 mm).

  - Material: ___________________.

  - Finish: (Architect to specify).

- Wood Handrail: (Architect to specify)

1. Profile: Part # 597, CRL-Blumcraft 1-3/4 inches x 2-3/4 inches (44.5 x 69.9 mm)

2. Profile: Part # 580, CRL-Blumcraft round 2 inches (50.8 mm) diameter.

3. Profile: Part # 572, CRL-Blumcraft 2 inches x 4-1/2 inches (50.8 x 114.3 mm)

4. Profile: Part # WD15, round 1-1/2 inches (38.1 mm) diameter

5. Profile: Part # WD20, round 2 inches (50.8 mm) diameter

6. Wood Type: _________________.

- Fasteners: Types and sizes indicated in shop drawings.

- For concrete attachment, hole size in base shoe is to be 9/16” (14.3 mm), counter bore 7/8” (22.2 mm) x depth ½” (12.7 mm), center-to-center spacing of holes is 12” (304.8mm). Use Hilti HSL3 Expansion Anchors 3-3/4" (95 mm) long CRL Part # EBA334, Washer is included.

- For steel attachment, hole size in base shoe is to be 9/16” (14.3 mm), counter bore 7/8” (22.2 mm) x depth ½” (12.7 mm), center-to-center spacing of holes is 12” (304.8mm). Use ½” – 13 x 1 stainless steel socket head cap screw CRL Part # SHCS12X1.

- Sill Angles for Tempered Glass Railing Assemblies: Steel angle profiles conforming to ASTM A 36, with anchoring devices, sizes indicated in shop drawing of section 05 5000, drilled and tapped for fastener types, sizes, and spacing indicated.

  - FABRICATION

- Fabricate handrail assembly components to lengths and configurations complying with shop drawings.

- Machine joint edges smooth and plane to produce hairline seams when site assembled; supply concealed sleeve connectors for joints.

- Isolate dissimilar metals to prevent electrolytic action by applying primer to concealed surfaces of metal components.

PART 3	INSTALLATION

3.1

- Install handrails in accordance with manufacturer’s recommended installation instructions and approved shop drawings.

  - CLEANING

- Clean glazing surfaces after installation, complying with requirements contained in the manufacturer’s instructions. Remove excess glazing sealant compounds, dirt or other substances.

- Remove protective films from metal surfaces.

- Clean railing surfaces with clean water and mild detergent. Do not use abrasive chemicals, detergents, or other implements that may mar or gouge the material.

  - PROTECTION

- Institute protective measures required throughout the remainder of the construction period to ensure that all the materials do not incur any damage or deterioration.

- Repair components damaged by subsequent construction activities in accordance with manufacturer’s recommendations; replace damaged components that cannot be repaired to Architect’s acceptance.

END OF SECTION