C.R. Laurence Co., Inc.		Distributed by ZeroDocs.com

SECTION 05 57 13 – GLAZED DECORATIVE METAL RAILINGS

GENERAL

SUBMITTALS

Action Submittals:

Shop Drawings: Illustrate products, installation, and relationship to adjacent construction.

Product Data: Manufacturer’s descriptive data and product attributes.

Informational Submittals:

Certificate of Compliance: Certification that installed products meet specified design and performance requirements.

Notice of Acceptance (NOA): Current NOA issued by Miami-Dae County Building Code Compliance Office.

SYSTEM DESCRIPTION

Design Requirements:

System design to be performed by qualified professional engineer licensed in State of [____].

QUALITY ASSURANCE

Fabricate railings in accordance with ASTM E935.

Railing System: Tested and approved for use in High Velocity Hurricane Zones as required by Florida Building Code..

Installer Qualifications: Firm specializing in work of this Section with minimum [2] [__] years’ experience.

Mockup: [Minimum [4] [__] feet long x full height.] [____.] Approved mockup [may] [may not] remain as part of the Work.

PRODUCTS

MANUFACTURERS

Contract Documents are based on Turboflex Adjustable All Glass Railing System by C.R. Laurence Co., Inc. www.crlaurence.com

Substitutions: [Refer to Division 01.] [Not permitted.]

SYSTEMS

Minimum Design Loads:

[50] [__] pounds per linear foot applied in any direction at top, transferred via attachments and supports to building structure.

Concentrated [200] [__] pound load applied in any direction at any point along top, t04 or 3ransferred via attachments and supports to building structure.

Maximum deflection under loading: [L/120.] [__.]

Glass Height Adjustment: Plus or minus 1/8 inch.

Glass Panel Tilt Adjustment: Plus or minus 2 degrees.

MATERIALS

Aluminum Extrusions: ASTM B221, 6005-T52 alloy and temper.

Aluminum Sheet: ASTM B209/B209M.

Stainless Steel: ASTM A240/A240M or ASTM A666/A666M, Type 304 or 316.

Glass: ASTM C1048, nominally [1/2] ]5/8] [3/4] inch thick [clear] [[____] tinted] tempered with exposed edges polished, by Oldcastle Building Envelope.

- **** OR ****

Glass: ASTM C1172 and ANSI Z97.1, nominally [9/16] [21/32] [11/16] [13/16] [27/32] inch thick [clear] [[____] tinted] laminated with exposed edges polished, with SG or ionoplast interlayer, by Oldcastle Building Envelope.

FABRICATION

Railing Height: [3’-6”] [__] from finished floor.

Mounting: [Surface.] [Fascia.]

Base Shoe: Extruded aluminum [clad with [stainless steel.] [aluminum.]]

Top Cap: Stainless steel, [radiused] [square] [low profile] type.

Fabricate railings with uniform ½ inch wide butt joints between glass panels.

FINISHES

Aluminum: [Mill finish.] [AAMA 611, Architectural Class I anodized, clear.] [AAMA 2604 powder coated, matte black.]

Stainless Steel: NAAMM AMP 503; [No. 4 satin.] [No. 8 polished.]

EXECUTION

INSTALLATION

Install in accordance with manufacturer's instructions and approved Shop Drawings.

Installation Tolerances:

Maximum variation from plumb or level: [1/8] [__] inch in 3 feet or [1/4] [__] inch in any 10 feet, whichever is less.

END OF SECTION