SECTION 08 41 13 – ALUMINUM-FRAMED ENTRANCE AND STOREFRONTS

GENERAL

SUBMITTALS

Action Submittals:

Shop Drawings: Illustrate products, installation, and relationship to adjacent construction.

Product Data: Manufacturer’s descriptive data and product attributes for entrances, door hardware, and storefronts.

Samples: [Selection samples.] [Verification samples.]

Informational Submittals:

Certificate of Compliance: Certification that installed products meet specified design and performance requirements.

SYSTEM DESCRIPTION

Entrance and Storefront System: Ultra narrow stile with insulating glazing, thermally broken, center glazed.

Design Requirements:

Design wind pressure: In accordance with [ASCE 7.] [Building Code.] [____.]

Movement: Ambient temperature range of [120] [__] degrees F and a surface temperature range of [170] [__] degrees F.

System design to be performed by qualified professional engineer licensed in State of [____].

Performance Requirements:

AAMA Performance Class: CW-PG30-LW.

Air infiltration: Maximum 0.22 CFM per square foot infiltration and maximum 0.15 CFP per square foot exfiltration, tested to AASTM E283 at 1.57 PSF.

Water penetration: No uncontrolled water, tested to ASTM E331.

System cycling: 1,000,000 cycles, tested to AAMA 920.

Uniform load deflection:

Peak deflection at center mullion: 0.561 inch at plus 25 PSF; 0.679 inch at plus 30 PSF.

Peak deflection at center mullion: 0.699 inch at minus 25 PSF; 0.786 inch at minus 30 PSF.

Uniform load permanent set at center mullion: Maximum 0.01 inch at plus 45.1 PSF; maximum 0.13 inch at minus 45 PSF.

Vertical load resistance: Pass 250 LBF load, tested to AAMA 925 with maximum 0.068 inch peak deflection and maximum 0.028 inch permanent deflection.

Forced entrance resistance: Pass AAMA 1304 at 300 pounds.

Thermal resistance: [__], tested to ANSI/NFRC 100-200-CMA, ANSI/NFRC 102, and AAMA 507.

Acoustic: 32 OITC, 37 STC, tested to ASTM E90 and E1425 with insulating glass units consisting of ¼ inch thick monolithic glass, 3/8 inch airspace, and 3/8 inch laminated glass.

QUALITY ASSURANCE

Installer Qualifications: Firm specializing in work of this Section with minimum [2] [__] years’ experience.

Handle products in accordance with AAMA CW-10.

PRODUCTS

MANUFACTURERS

Contract Documents are based on Entice HP+ Series by C.R. Laurence. www.crlaurence.com

Substitutions: [Refer to Division 01.] [Not permitted.]

MATERIALS

Aluminum Extrusions: ASTM B221, 6061-T5 or T6 alloy and temper.

Stainless Steel: ASTM A666, Type 304.

- **** OR ****

Brass: ASTM B36/B36M, Alloy No. C26000.

Glass: Specified in Section [08 80 00.] [__ __ __.]

Door Hardware:

Hinges: Continuous type, aluminum, 760 pound rated.

Locks:

Manufacturer’s standard.

Round throw deadbolt in continuous bottom fitting.

Function: Key outside x thumbturn inside with end load capability.

Cylinders: Specified in Section [08 71 00.] [__ __ __.]

Lock handles: C.R. Laurence [DB100.] [DB110.] [DB130.] [DB140.] [DB150.] [DB160.] [DB170.]

Exit devices: Concealed vertical rod type, 1-1/4 inch diameter; C.R. Laurence [PA100.] [PA110.]

Push and pull: [____.] [To be selected from manufacturer’s full product range.]

Overhead concealed closers: C.R. Laurence LCN 2030 Series.

Electronic egress control handles: C.R. Laurence [EG100.] [EG110.]

Electric strike: ESK (1)(2).

Weatherstripping: Fin type, continuous and replaceable.

Threshold: Extruded aluminum, one piece per door opening, ribbed surface.

ACCESSORIES

Anchors: Series 300 stainless steel.

FABRICATION

System Components:

Stiles: 1-5/8 inches wide x 2-7/8 inches deep.

Top rail: 4 inches high.

Bottom rail: [4] [10] inches high.

Intermediate verticals: 2 inch face dimension x 2-11/16 inch depth

FINISHES

Aluminum: AAMA 611, Architectural Class I anodized, [clear.] [black.] [bronze.]

**** OR ****

Aluminum: Powder coat, [____] color [to be selected from manufacturer’s full color range.]

**** OR ****

Aluminum: AAMA 2605, fluoropolymer coating, minimum 70 percent PVDF resins, [____] [custom] color [to be selected from manufacturer's full color range].

**** OR ****

Stainless Steel: [Brushed.] [Polished.]

**** OR ****

Brass: [Brushed.] [Polished.]

EXECUTION

INSTALLATION

Install in accordance with manufacturer's instructions and approved Shop Drawings.

Installation Tolerances:

Maximum variation from plumb or level: [1/8] [__] inch in 3 feet or [1/4] [__] inch in any 10 feet, whichever is less.

Maximum misalignment of members abutting end to end: [1/32] [__] inch.

Sealant space between system and adjacent construction: [1/2] [__] inch plus or minus [1/8] [__] inch.

END OF SECTION